CCNA Lab Report

Basic Network Configuration & Troubleshooting

Cisco Packet Tracer

# 1. Lab Objective

This lab demonstrates the configuration and troubleshooting of a small two-subnet network using Cisco Packet Tracer. The topology consists of two Layer 2 switches (S1, S2), one router (R1), and two PCs. The objectives are:

- Correctly assign IPv4 addresses and subnet masks to all devices

- Configure default gateways on PCs and switches

- Identify and fix misconfigured IP addresses and subnet masks

- Configure switchports with correct access mode and spanning-tree portfast

- Verify end-to-end connectivity using ICMP ping

# 2. Network Topology

The logical topology is a linear chain: PC1 — S1 — R1 — S2 — PC2, with R1 providing inter-subnet routing between the two /24 networks.

[PC1]──────[S1]──────[R1 Gi0/0/0]──[R1 Gi0/0/1]──────[S2]──────[PC2]

192.168.1.1/24       192.168.1.126/24  192.168.2.254/24          192.168.2.1/24

GW: 192.168.1.126    (VLAN1: 192.168.1.125)  (VLAN1: 192.168.2.122)

# 3. Final IP Addressing Table

The table below shows the corrected and verified IP configuration for all devices.

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| PC1 | NIC (Fa0) | 192.168.1.1 | 255.255.255.0 | 192.168.1.126 |
| PC2 | NIC (Fa0) | 192.168.2.1 | 255.255.255.0 | 192.168.2.254 |
| S1 | VLAN1 | 192.168.1.125 | 255.255.255.0 | 192.168.1.126 |
| S2 | VLAN1 | 192.168.2.122 | 255.255.255.0 | 192.168.2.254 |
| R1 | Gi0/0/0 | 192.168.1.126 | 255.255.255.0 | N/A |
| R1 | Gi0/0/1 | 192.168.2.254 | 255.255.255.0 | N/A |

# 4. Configuration Issues Found & Fixes Applied

## Issue 1 — S1: Subnet Mask Mismatch

S1's VLAN1 interface had a /25 mask (255.255.255.128) while R1's Gi0/0/0 used /24 (255.255.255.0). Both devices are on the same physical segment and must use identical masks.

Before:

interface Vlan1

ip address 192.168.1.125 255.255.255.128  ! /25 — WRONG

Fix applied:

S1(config)# interface vlan 1

S1(config-if)# ip address 192.168.1.125 255.255.255.0

## Issue 2 — S2: IP Address on Wrong Subnet

S2's VLAN1 was assigned 192.168.1.253/25, placing it on the 192.168.1.128/25 subnet. Its default gateway was 192.168.2.254 — on a completely different network. S2 could never reach its gateway.

Before:

interface Vlan1

ip address 192.168.1.253 255.255.255.128  ! Wrong subnet

ip default-gateway 192.168.2.254           ! Unreachable

Fix applied:

S2(config)# interface vlan 1

S2(config-if)# ip address 192.168.2.122 255.255.255.0

S2(config-if)# no shut

S2(config)# ip default-gateway 192.168.2.254

## Issue 3 — PC2: Wrong Default Gateway

PC2 was physically connected to S2 (on the 192.168.2.0/24 network) but was configured with IP 192.168.1.2/24 and gateway 192.168.2.254. The IP address was on the wrong subnet relative to PC2's physical location.

Fix applied:

PC2 Desktop > IP Configuration:

IP Address:      192.168.2.1

Subnet Mask:     255.255.255.0

Default Gateway: 192.168.2.254

## Issue 4 — S2 Gi1/0/2: Dynamic Auto Mode

S2's Gi1/0/2 (PC-facing port) was in 'dynamic auto' mode with trunking negotiation enabled. This is a security risk and should be explicitly set to access mode on end-device ports.

Fix applied:

S2(config)# interface GigabitEthernet1/0/2

S2(config-if)# switchport mode access

S2(config-if)# switchport access vlan 1

S2(config-if)# spanning-tree portfast

# 5. Final Device Configurations

## R1

interface GigabitEthernet0/0/0

ip address 192.168.1.126 255.255.255.0

duplex auto

speed auto

!

interface GigabitEthernet0/0/1

ip address 192.168.2.254 255.255.255.0

duplex auto

speed auto

## S1

interface GigabitEthernet1/0/1

switchport mode access

spanning-tree portfast

!

interface GigabitEthernet1/0/2

switchport mode access

!

interface Vlan1

ip address 192.168.1.125 255.255.255.0

!

ip default-gateway 192.168.1.126

## S2

interface GigabitEthernet1/0/1

switchport mode access

!

interface GigabitEthernet1/0/2

switchport mode access

spanning-tree portfast

!

interface Vlan1

ip address 192.168.2.122 255.255.255.0

!

ip default-gateway 192.168.2.254

# 6. Connectivity Verification

End-to-end connectivity was verified using the Packet Tracer Simulation mode. ICMP packets were sent between PC1 and PC2 to confirm cross-subnet routing through R1.

| Source | Destination | Protocol | Result |
| --- | --- | --- | --- |
| PC1 (192.168.1.1) | PC2 (192.168.2.1) | ICMP | SUCCESS |
| PC2 (192.168.2.1) | PC1 (192.168.1.1) | ICMP | SUCCESS |

Both directions of ICMP traffic traversed the full path: PC → Switch → R1 → Switch → PC, confirming that routing between the 192.168.1.0/24 and 192.168.2.0/24 subnets is functioning correctly.

# 7. Summary

This lab reinforced the following key networking concepts:

- IP addressing and subnetting — all devices must use consistent masks on shared segments

- Default gateway configuration — a device's gateway must be reachable on the same subnet

- Switchport modes — end-device ports should be explicitly set to access mode to prevent VLAN hopping

- Spanning-tree PortFast — reduces convergence delay on PC-facing ports

- Using show commands (show ip int brief, show cdp neighbors, show int switchport) to diagnose misconfigurations